GRADIOMETER PRO v2.0

Firmware Engineering Reference

ESP32 · FLC100 Fluxgate · ADS1115 · LVGL · FreeRTOS

ESP-IDF v5.5.3  ·  LVGL v8  ·  EEZ Studio

Session Reference — April 2026

# 1. Hardware Platform

| Component | Part / Value | Notes |
| --- | --- | --- |
| MCU | ESP32 dual-core 240MHz | Core0=signal/BT, Core1=UI/BLE |
| Sensors | 2× FLC100 fluxgate | 45 cm apart, wooden rod / PVC tube |
| ADC | ADS1115 differential | AIN0=lower sensor, AIN1=upper |
| Display | ILI9341 320×240 TFT | XPT2046 touch controller |
| BT Classic | HC-05 UART | GPIO17/21 → OKM Visualizer3D (PC) |
| BLE | ESP32 internal | CSV notifications → Android/iOS |
| Buzzer | GPIO26 LEDC PWM | LEDC_CHAN_0, TIMER_0, 13-bit |
| Button | GPIO16 | Scan trigger + device cal control |

### Pin Map

| Signal | GPIO | Signal | GPIO |
| --- | --- | --- | --- |
| TFT_DC | 2 | TFT_RST | 4 |
| TFT_CS | 5 | SPI_CLK | 18 |
| SPI_MISO | 19 | SPI_MOSI | 23 |
| T_CS | 25 | I2C_SCL | 22 |
| I2C_SDA | 27 | BT_TX | 17 |
| BT_RX | 21 | BUZZER | 26 |
| SCAN_BTN | 16 | — | — |

# 2. Firmware Framework

| Item | Value |
| --- | --- |
| ESP-IDF | v5.5.3 |
| RTOS | FreeRTOS (dual-core pinned tasks) |
| UI Framework | LVGL v8 + EEZ Studio generated screens |
| LVGL Buffer | Single 10-line buffer (DRAM optimised, BLE fit) |
| Build | idf.py + ninja → ~567 KB / 1 MB flash (45% free) |
| Toolchain | xtensa-esp-elf, Windows host |
| Flash partitions | Bootloader 9% ¦ App 55% ¦ NVS / OTA reserved |

# 3. Task Architecture

| Task | Core | Prio | Stack | Role |
| --- | --- | --- | --- | --- |
| adc_task | 0 | 6 | 4096 B | 860SPS decimation + boost mode |
| signal_task | 0 | 5 | 4096 B | DSP pipeline + Kalman + spatial |
| bt_task | 0 | 4 | 2048 B | HC-05 UART binary to OKM PC |
| ble_logger | 1 | 3 | 4096 B | BLE GATT CSV → mobile |
| lvgl_task | 1 | 5 | 6144 B | LVGL tick + lv_timer_handler |
| ui_event_task | 1 | 4 | 4096 B | Bridge backend ↔ EEZ UI |

### Queues

| Queue | Depth | Item Type | Direction |
| --- | --- | --- | --- |
| adc_queue | 4 | AdcSample_t (12B) | adc_task → signal_task |
| result_queue | 2 | ProcessedSample_t (30B) | signal_task → ui_event_task (overwrite) |
| event_queue | 8 | SysEventMsg_t | any → signal_task |
| bt_queue | 32 | BTMessage_t | signal_task → bt_task |
| ble_queue | 8 | BleLogPoint_t | signal_task → ble_logger |

# 4. Component Structure

| Path | Key Files | Responsibility |
| --- | --- | --- |
| core/ | gradiometer_types.h queue_manager.c signal_processor.c signal_task.c | All shared types (zero deps) Queues lifecycle DSP pipeline Task orchestrator |
| drivers/ | adc_task.c ads1115_driver.c bluetooth_sender.c | ADC decimation+boost I2C differential reads HC-05 priority queue+retry |
| calibration/ | calib_engine.c | 2-phase Welford soil calibration |
| device_calibration/ | device_cal.c devcal_common.c devcal_phase1-5.c | 5-phase NVS calibration wizard CRC32 atomic save |
| ble_logger/ | ble_data_logger.c | GATT server + CSV notifications |
| display/ | ili9341_driver.c lvgl_port.c touch_calibration.c | TFT driver Single-buffer LVGL port 9-point affine touch cal |
| modes/ | sensitivity_manager.c | AUTO engine + PGA + FSM hysteresis |
| ui/ | ui_event_task.c actions.c screens.c | 2400-line bridge + audio engine 19 registered callbacks EEZ Studio generated |

# 5. DSP Signal Processing Pipeline

## Pipeline A — Screen + Buzzer

| Stage | Implementation | Notes |
| --- | --- | --- |
| 1. Moving Average | O(1) running sum, window 4–64 | Window from sensitivity mode |
| 2. Outlier Gate | σ-rejection vs local MA | Noise updated from RAW (bug fixed) |
| 3a. Kalman 1D | Adaptive Q (1–8×), fixed R | Toggleable; Q from device cal |
| 3b. EMA fallback | α from sensitivity table | When Kalman disabled |
| 3c. Spatial filter | Pure gradient dG (bug fixed) | Scan mode only; experimental |
| 4. Baseline subtract | Dual-track adaptive | baseline_fixed + adaptive drift |
| 5. Post-Kalman EMA | α×0.3 light smoothing | Only when Kalman ON |
| 6. Noise gate | Threshold from device cal | Suppress sub-threshold output |
| 7. Scale output | [0..1024] for screen | GRAD_OUTPUT_CENTER = 512 |

## Pipeline B — HC-05 → OKM Visualizer3D

Kalman output → Adaptive DC drift removal (α=0.0005, ~100s) → Fixed linear mapping using k×noise_floor range → clamp [0..1] → ×1024 → uint16 → UART.

k factor per soil type: PRISTINE=3 · CLEAN=4 · MINERAL=5 · NOISY=6 · EXTREME=7

## Oversampling

| Mode | Normal | Boost (+18/+24 dB) |
| --- | --- | --- |
| Live | 8 samples | 64 samples |
| Manual/Auto | 32 samples | 256 samples |
| Inter-sample | 2ms (860SPS settling) | — |

## Sensitivity Modes

| Mode | Window | α EMA | σ Outlier | Noise Gate |
| --- | --- | --- | --- | --- |
| AUTO (→MEDIUM) | 16 | 0.30 | 2.5 | 0.5 |
| VERY_HIGH | 4 | 0.60 | 1.5 | 0.1 |
| HIGH | 8 | 0.45 | 2.0 | 0.3 |
| MEDIUM | 16 | 0.30 | 2.5 | 0.5 |
| LOW | 32 | 0.15 | 3.5 | 1.0 |
| VERY_LOW | 64 | 0.08 | 4.5 | 2.5 |

# 6. Calibration Systems

## 6.1 Soil Calibration (2-phase, before each scan)

| Phase | Duration | What it measures | Output |
| --- | --- | --- | --- |
| Phase 1 — Static | 20s still | baseline_fixed + noise_static (Welford) | Single beep at end |
| Phase 2 — Dynamic | 40s slow walk | noise_dynamic = std(signal − local_MA_16) | Double beep at start |

noise_floor_final = max(noise_static, noise_dynamic) · kalman_R = noise_floor² · Outlier rejection: |residual| > 3σ → skip

## 6.2 Device Calibration (5-phase, GPIO16 button)

| Phase | Duration | Measures |
| --- | --- | --- |
| 1 — Sensor Offset + Noise | 60s + 10s warmup | noise_floor, kalman_R |
| 2 — Sensor Matching | 160 samples/step | gradient range (N+S bar magnet), kalman_Q |
| 3 — Horizontal Compass | 16s/direction × 4 | heading corrections N/E/S/W |
| 4 — Tilt | 24s/axis × 2 | tilt_ns, tilt_ew, threshold |
| 5 — Final Noise Floor | 30s | kalman_Q_final, R_final, detection_limit |

NVS save order (power-loss safe): blob → CRC32 → cal_version → device_valid=1.  Touch calibration namespace is NEVER touched during device cal reset.

# 7. AUTO Sensitivity Engine

## Architecture

sensitivity_manager.c is a pure decision layer — it never touches ADC samples or DSP. signal_task calls sens_manager_auto_update() after every sp_process().

| Layer | File | Role |
| --- | --- | --- |
| DSP | signal_processor.c | Processes samples, computes SNR/stability/noise_variance |
| Decision | sensitivity_manager.c | AUTO engine: EMA + FSM + hysteresis → selects mode |
| Router | signal_task.c | Calls auto_update(), applies pending mode changes |
| Hardware | ads1115_driver.c | PGA change on ADS1115 from apply_mode() |

## Decision Algorithm

| Condition | Target Mode |
| --- | --- |
| noise_ema ≥ 30 LSB | VERY_LOW (forced) |
| noise_ema ≥ 12 LSB | LOW (forced) |
| noise_ema < 12 AND SNR ≥ 6.0 AND stability ≥ 85 | VERY_HIGH |
| noise_ema < 12 AND SNR ≥ 3.0 AND stability ≥ 70 | HIGH |
| else | MEDIUM (fallback) |

Protections: EMA alpha=0.05 on noise_floor (TC≈4s) · noise_floor clamped ≥ 0.02 · Stability gate: no decision if !stable · Time lock: 1200ms between changes · FSM: step one level at a time (no jumps) · Mutex protects apply_mode() from Core0/Core1 race conditions.

FSM Step-down: VERY_HIGH→HIGH only · HIGH→MEDIUM only · LOW→MEDIUM only · MEDIUM→HIGH only.

# 8. SNR Panel + Smart Detection Layer

## 8.1 SNR Panel (update_snr_panel)

| SNR Range | Color | Label | Confidence |
| --- | --- | --- | --- |
| < 1.0 | #2ECC71 Green | Clear soil | 0–20% |
| 1–2 | #F1C40F Yellow | Weak signal | 20–40% |
| 2–4 | #E67E22 Orange | Possible target | 40–80% |
| > 4 | #E74C3C Red | Strong target! | 80–100% |
| !stable | #7F8C8D Grey | Moving... | 0% |

Peak Hold: time-based decay powf(0.98, dt×10) per 100ms · Jerk detection: adaptive threshold = clamp(2×noise_floor, 0.5, 5.0) · On jerk: peak ×0.50 · On normal movement: peak ×0.85 · Hysteresis zones: ±0.2 thresholds prevent color flicker.

## 8.2 Smart Detection (smart_detection_update)

5 protection layers applied in order:

| Layer | Mechanism | Parameter |
| --- | --- | --- |
| A. Momentum EMA | EMA on Δdeviation, α=0.30 · ×0.20 when !stable | s_det_momentum, ceiling ±3.0, dead-band 0.05 |
| B. Void consecutive | Requires N sequential negative samples | 3 samples (quiet) · 4 samples (noise_floor > 5 LSB) |
| C. Confidence hysteresis | No Metal↔Void switch < 40% confidence | DETECTION_TYPE_HYST_CONF = 40% |
| D. Time hysteresis | No type switch < 300ms after last change | DETECTION_TIME_HYST_MS = 300 |
| E. Temporal Hold + fade | Hold last result 120–550ms (scales with confidence) | EMA fade ×0.92 per 200ms · floor 10% · dt_norm clamp 3.0 |

Colors — Metal: Green(30%) → Yellow(60%) → Red. Void: light blue → mid blue → dark blue.

# 9. Professional Audio Engine

## Design

LEDC init once (not per-update). All updates via ledc_set_duty() only — saves ~45μs/update vs full ledc_timer_config().

| Mode | Freq | Pattern | Duty | Controls |
| --- | --- | --- | --- | --- |
| METAL | 800 Hz | Pulse (beep-beep) | f(confidence) 800–5120 | Rate: 500ms − SNR²×15 − vel×50 [clamp 80–500ms] |
| VOID | 200 Hz | Continuous tone | EMA-smoothed 600–4000 | Soft attack α=0.15, snap-to-zero on mute |
| SILENCE | — | No output | 0 | Hysteresis: ON at dev>12, OFF at dev<8 LSB |

Velocity (approach speed): vel = SNR − prev_SNR · clamp [0..1.5] · contributes −vel×50ms to pulse interval.

Quadratic SNR: pulse_interval = 500 − SNR²×15 − vel×50 · anti-machine-gun floor: 80ms.

# 10. BLE Data Logger

| Item | Value |
| --- | --- |
| Device name | "Gradiometer BLE" |
| Service UUID | 4AFAFADE-1234-11EF-8A1C-2B7E8F6A9C3E |
| Properties | READ + NOTIFY + CCCD |
| CSV fields | timestamp_ms, scan_mode, step, gradient_raw, gradient_filtered, baseline, deviation, bt_value, snr, stability, soil_type, sensitivity, boost, kalman, heading, noise_floor, quality_note |
| Rate limit | 20 points/sec (50ms min between points) |
| DRAM savings | GATTC disabled (−15KB) · SMP disabled (−8KB) · Dynamic BLE memory |
| Activation | data_via_ble switch ON + scan active + BLE client subscribed |

# 11. Bluetooth Sender (HC-05 → OKM)

Architecture: independent bt_sender_task with priority queue. Scan points use xQueueSendToFront() to bypass live messages.

Data integrity fix: current_step++ only increments after confirmed send. On queue full: retry with 20ms timeout. On both failures: ESP_LOGE + return false → ui_event_task resets countdown for retry.

| Message type | Queue position | Retry |
| --- | --- | --- |
| Scan point | Front (priority) | 10ms + 20ms fallback · no counter if both fail |
| Live update | Back (normal) | Best-effort, dropped if full |

# 12. Bug Fixes Applied This Session

| Bug | File | Severity | Fix |
| --- | --- | --- | --- |
| ma_sum not reset on mode change → false target spike | signal_processor.c | 🔴 Critical | sp->ma_sum = 0.0f in sp_set_sensitivity() |
| update_noise(gated) feedback loop → system lockup | signal_processor.c | 🔴 Critical | update_noise(raw, ma_out) — always use raw measurement |
| Spatial filter baseline halved on flat ground → ghost void | signal_processor.c | 🟡 Medium | Pure gradient output: output = dG_sym |
| s_sig.bt assigned before memset → silent overwrite | signal_task.c | 🟡 Medium | Removed pre-memset assignment; comment added |
| vTaskDelay(180ms) inside UI_LOCK → LVGL freeze | ui_event_task.c | 🔴 Critical | Moved second Phase2 beep outside UI_LOCK in main loop |
| bt_enqueue_scan: current_step++ regardless of send success | bluetooth_sender.c | 🔴 Critical | Increment only after confirmed send; return false on drop |
| Static s_auto not reset on re-init | sensitivity_manager.c | 🟡 Medium | memset(&s_auto, 0) inside sens_manager_init() |
| FSM step-jump: LOW could skip to VERY_HIGH directly | sensitivity_manager.c | 🟡 Medium | Added LOW case; all transitions limited to ±1 level |
| Race condition: apply_mode() unprotected across cores | sensitivity_manager.c | 🔴 Critical | SemaphoreHandle_t mutex in struct; LOCK/UNLOCK in apply_mode() |
| SENS_MODE_INFO table had 4 entries, enum has 6 | sensitivity_manager.c | 🟡 Medium | Expanded to [6] with VERY_HIGH and VERY_LOW |
| Spatial filter inverted exit logic in VERY_HIGH FSM | sensitivity_manager.c | 🟡 Medium | Rewrote FSM: "Can I stay?" logic corrected |
| GRAD_CALIB_DURATION_MS = 10000 (was 10s, should be 60s) | gradiometer_types.h | 🔴 Critical | Fixed to 60000u (Phase1:20s + Phase2:40s) |
| ProcessedSample_t: raw_differential first → float misaligned | gradiometer_types.h | 🟢 Minor | Reordered: 4B fields first, 2B last (Xtensa LX6 optimization) |
| pipeline_bt param named kalman_out (misleading) | signal_processor.c | 🟢 Minor | Renamed to signal_in with clarifying comment |
| Unused #include sensitivity_manager.h in UI task | ui_event_task.c | 🟢 Minor | Removed (compilation guard — UI must not call sens_manager directly) |

# 13. Pending / Future Work

| Item | Priority | Notes |
| --- | --- | --- |
| ICM-20948 IMU integration | Medium | Tilt comp (Phase 4 already has tilt_ns/ew) · Vibration gate · Better heading Phase 3A |
| seconds_numbers range up to 20s | Low | Currently capped at 10s |
| soil_type getter for BLE logger | Low | Currently sends 0 |
| SYS_EVT_SENS_USER_REQ event | Low | Distinguish user request from auto engine internally in signal_task — not urgent, Mutex handles race |
| sentinel -1.0f → proper event | Low | TECHNICAL DEBT: Phase2 transition via ProcessedSample_t.stability sentinel violates SRP |
| bt_sender_deinit vTaskDelete | Low | Unsafe if task runs uart_write_bytes — use volatile run flag for clean shutdown |
| ADS1256 24-bit ADC upgrade | Future | For deeper target detection |
| Baseline 45cm → 70–100cm | Future | Requires full device recalibration |
| Field Validation Layer | Future | VALID/UNCERTAIN/NOISE per scan point — in signal_task.c not UI |
| Spatial Pattern Engine | Future | Width tracking for Metal(narrow) vs Void(wide) — scan mode only, signal_task.c |

# 14. NVS Storage

| Namespace | Key | Type | Notes |
| --- | --- | --- | --- |
| "dev_cal" | profile_v2 | blob (DeviceProfile_t) | Written FIRST |
| "dev_cal" | dev_crc | uint32 (CRC32) | Written SECOND |
| "dev_cal" | cal_version | uint8 = 2 | Written THIRD |
| "dev_cal" | device_valid | uint8 = 1 | Written LAST (gate) |
| "touch_cal" | coeffs_v2 | TouchCalCoeffs_t | NEVER touched by device cal reset |

Boot logic: device_valid ≠ 1 OR version mismatch → run 5-phase wizard. Touch calibration is independent and protected.

# 15. Build Status & Key File Metrics

| File | Lines | Status |
| --- | --- | --- |
| ui_event_task.c | ~2440 | ✓ Audio Engine + Smart Detection + SNR Panel |
| signal_processor.c | ~790 | ✓ 3 critical DSP bugs fixed this session |
| signal_task.c | ~550 | ✓ memset order fix + TECHNICAL DEBT comment |
| sensitivity_manager.c | ~460 | ✓ Auto Engine + Mutex + FSM + 6-entry table |
| bluetooth_sender.c | ~310 | ✓ Data integrity fix (counter after confirm) |
| app_main.c | ~660 | ✓ g_sens_mgr global + sens_manager_init() |
| devcal_common.c | ~900 | — Not modified this session |
| gradiometer_types.h | ~255 | ✓ 60s calib duration + struct alignment |

Last successful build: ~567KB / 1MB flash · Bootloader 9% · App 55% free

Platform: Windows · ESP-IDF v5.5.3 · idf.py build -- -j 6